The Ultimate POCSAG decoder v1.03d

----------------------------------

Topics

------

1.0	About

1.1	Release notes

1.2	Features

1.3	Compiling

1.4	Configuration

1.5	Output modules

1.6	Output configuration

1.7     Filters

1.8	Error highlighting

1.9	Usage hints

2.0	Known issues

2.1	Contact

2.2	Acknowledgements

2.3	License

2.4	Legal stuff

1.0 About

---------

This program decodes the Post Office Code Standardization Advisory Group (POCSAG)

pager protocol.

This project originally began as a Perl script in January of 2008 when I realized

that there were no working POCSAG decoders for Linux, and I badly wanted one.

It has since been rewritten and improved to the C only version you're holding

in your hands right now.

1.1 Release notes

-----------------

This program is being made available on an "as is" basis.

There may be bugs I have not yet discovered and this documentation is

probably lacking in some respects.

I would like to thank everybody who has provided feedback and suggestions

after the first version was released. You know who you are.

1.2 Features

------------

* Soundcard input via OSS (usually /dev/dsp).

* Server mode: One instance of the decoder can serve audio and raw POCSAG streams to

other instances of the decoder connecting as clients (more on this later).

* Telnet server.

* Logging of raw POCSAG codewords to disk.

* Error highlighting.

* Noise smoothing (reduces error rate when input signal is weak and/or noisy).

* Statistics logging.

* Displaying of raw POCSAG codewords.

* Filtering based on keywords, RIC, and function bits.

* Logging of decoded messages to:

Console

Disk

MySQL

IRC

Windows machines (popup using the messenger service)

Telnet server

E-mail

External applications

1.3 Compiling

-------------

./configure

make

If you want MySQL support, please read mysql-notes.doc before running configure.

It has been tested and is known to compile and run on:

Debian 4, 6 and 7

Ubuntu 12

FreeBSD 9.0

It will probably not run on any OS that does not support the Open Sound System (OSS).

Support for other sound interfaces may (or may not) be included in future releases.

1.4 Configuration

-----------------

The configuration can be provided either through a file or through the command line.

A configuration file is the preferred method.

The configuration file is divided into two main categories: the main section

and the output sections.

For the configuration file, the format is:

[main]

option_name1=value

[output_config1]

option_name2=value

option_name must be specified in lowercase, and there must not be any spaces

on either sides of the = sign.

The equivalent command line argument syntax of the above is

--option_name1=value --output_config1,option_name2=value

Under the [main] section, the following options are available:

input-mode=mode

---------------

Used in conjuction with input-source. Selects the input mode.

Possible values for mode are:

dsp-pcm

tcp-pcm

tcp-raw

file-raw

Default value: dsp-pcm

input-source=source

-------------------

If input-mode is dsp-pcm, source will be an OSS device name (typically /dev/dsp)

If input-mode is tcp-pcm or tcp-raw, source will be in the format ip:port

If input-mode is file-raw, source will be previously logged raw POCSAG

codewords filename(s). Comma separation and wildcards are supported.

Example: input-source=127.0.0.1:9000 (if input-mode=tcp-pcm or tcp-raw)

Example: input-source=2011-01-*.pocsag 2011-03-*.pocsag (if input-mode=file-raw)

Note: If you use the "file-raw" mode on the command line and you wish to specify

multiple files (including ones with wildcards), you have to escape the space characters.

Example: --input-source=2011-01-*.pocsag\ 2011-03-*.pocsag

If you use input-source in the configuration file, no escaping is required.

When the program reads files in the "file-raw" mode, it will check the

timestamps of each POCSAG batch, so that currently processed batch

is not older than the previous one.

Example:

This will typically work: input-source=2011-01-01.pocsag 2011-01-02.pocsag

While this will probably generate an error message:

input-source=2011-01-02.pocsag 2011-01-01.pocsag

Default value: /dev/dsp

dsp-sample-rate=value

---------------------

If input-mode is dsp-pcm, this specifies the desired sampling rate that

the program will try to obtain from the sound device.

Default value: 48000

dsp-sample-bits=value

---------------------

If input-mode is dsp-pcm, this specifies the desired bits per samples that

the program will try to obtain from the sound device.

Possible values: 8 and 16

Default value: 16

dsp-input-mode=value

--------------------

If input-mode is dsp-pcm or dsp-tcp, this options decides which channels should be

decoded:

Possible values: left, right and stereo

Default value: stereo

bitrates=list

-------------

Specifies which bitrates you want to decode. List is a comma separated

list of bitrate values.

Example: bitrates=512,1200

Default value: 512,1200,2400

alphanumeric-bits=list

----------------------

Specifies which function bit(s) a message must have in order for it

to be decoded as an alphanumeric message.

Example: alphanumeric-bits=3

Possible values: 0,1,2 and 3.

Default value: 3

The numbers in this list must not overlap with those of numeric-bits.

numeric-bits

------------

Specifies which function bit(s) a message must have in order for it

to be decoded as a numeric message.

Example: numeric-bits=0,1,2

Default value: 0,1,2

The numbers in this list must not overlap with those of alphanumeric-bits.

pcm-server-password=STRING

--------------------------

Enables password protection for the PCM server.

pcm-server=yes

--------------

This allows the decoder to act as a server, serving its incoming PCM stream

to other instances of the decoder configured as clients.

Example configuration:

Server:

[main]

input-mode=dsp-pcm

input-source=/dev/dsp

pcm-server=yes

pcm-server-port=5000

pcm-server-password=password

Client:

[main]

input-mode=tcp-pcm

input-source=127.0.0.1:5000

input-password=password

It is also possible for a client to act as a server.

Example configuration:

Server:

[main]

input-mode=dsp-pcm

input-source=/dev/dsp

pcm-server=yes

pcm-server-port=5000

pcm-server-password=password

Client 1:

[main]

input-mode=tcp-pcm

input-source=127.0.0.1:5000

input-password=password

pcm-server=yes

pcm-server-port=5001

pcm-server-password=password2

Client 2:

[main]

input-mode=tcp-pcm

input-source=127.0.0.1:5001

input-password=password2

In this example, "Server" till receive its input from the sound device

at /dev/dsp. It will copy this stream to clients connecting on port 5000.

Client 1 will connect to Server on port 5000. Client 2 will connect to

Client 1 and receive a copy of the stream it's receiving from Server.

It is theoretically possible to create a long chain of decoders, each

feeding the other.

The only limitation is: If input-mode=tcp-raw, then the pcm-server

will not work, only raw-server (see below).

input-password=STRING

---------------------

Specifies what password to use if the current input mode

is tcp-pcm or tcp-raw.

This is only needed if a password has been enabled on the "remote" end

(pcm-server-password or raw-server-password).

raw-server-password=STRING

--------------------------

Enables password protection for the RAW server.

raw-server=yes

--------------

This allows the decoder to act as a server, serving raw POCSAG codewords

to other instances of the decoder configured as clients.

Example configuration:

Server:

[main]

input-mode=dsp-pcm

input-source=/dev/dsp

raw-server=yes

raw-server-port=5000

raw-server-password=password

Client:

[main]

input-mode=tcp-raw

input-source=127.0.0.1:5000

input-password=password

Similar to the pcm-server, it's possible for a client where input-mode=tcp-raw

to also act as a raw-server. As mentioned above, when input-mode=tcp-raw,

pcm-server=yes will not work.

raw-logging=yes

---------------

This enables the logging of raw POCSAG codewords to disk.

Default value: no

raw-logging-error-threshold=value

---------------------------------

This value decides which filename to log a batch of raw POCSAG codewords to,

depending on its error level.

Example 1:

Assume the following:

G = Good codeword

C = Codeword with correctable errors

B = Bad codeword

For a batch to be considered valid, the following relation must hold:

count(G) + count(C) >= raw-logging-error-threshold

One POCSAG batch is 16 codewords.

raw-logging-error-threshold=16

Incoming batch: G G G G G G G G G G G G G G G G

Sum: 16 good codewords.

Result: 16 is equal to or greater than the raw-logging-error-threshold value.

Log the data to the filename specified by POCSAG_RAW_LOG_OK in pocsag.h

Incoming batch: G G G G G G G G G G G G G G G B

Result: 15 good codewords, one bad. This is less than the value specified

by raw-logging-error-threshold.

Log the data to the filename specified by POCSAG_RAW_LOG_ERR in pocsag.h

Incoming batch: G G G G C C C G G G G G G G G G

Sum: 13 good codewords + 3 codewords with correctable errors

count(G) + count(C) >= raw-logging-error-threshold

Result: Log the data to the filename specified by POCSAG_RAW_LOG_OK in pocsag.h

Default value: 16

ric-lookup=string

-----------------

This option allows you to assign strings to different pager addresses (or RIC,

Radio Identity Code).

When a RIC number that has been configured here is matched to an incoming message,

the assigned string is inserted into the output of the logging code.

Currently, this is implemented in all output modules except the MySQL one.

"string" is a comma separated list of values, where the value has two possible formats:

Format 1 = ric=string

Format 2 = channel:ric=string

If you specify a channel, the entry will only match if the RIC is coming in through

that specified channel.

For example, if dsp-input-mode=stereo, the left channel will be channel 0, the right will be

channel 1. If dsp-input-mode=left or right, that single input channel will be 0.

Examples:

[main]

ric-lookup=12345=FD,33333=User 1,44444=User 2

Assign the string "FD" to RIC 12345 coming in on any channel,

"User 1" to RIC 33333, etc.

The following is also valid:

[main]

ric-lookup=12345=FD,0:33333=User 1,1:44444=User 2

Assign the string "FD" to RIC 12345 coming in on any channel,

"User 1" to RIC 33333, but only if the message is coming in on channel 0.

"User 2" to RIC 44444, but only if the message is coming in on channel 1.

print-codewords=yes

-------------------

Prints the POCSAG codewords being decoded, including channel and bitrate.

Default value: no

print-codewords-eh=yes

----------------------

Uses ANSI codes to highlight errors in codewords.

Green = 1 or 2 correctable errors

Red = Uncorrectable error

Default value: yes

This assumes that print-codewords=yes

program-log=yes

---------------

Logs all program messages (except decoded messages)

to disk.

Default value: yes

program-logfile=filename

------------------------

This specifices the filename for the program log.

Used when program-log=yes

Default value: pocsag.log

stats=yes

---------

This will keep track of decoding statistics.

When enabled, you can view the statistics by

pressing CTRL-C.

Default value: yes

stats-file=filename

-------------------

Where to store decoding statistics.

Used when stats=yes

Default value: stats.dat

noise-smoothing=yes

-------------------

This feature attempts to reduce the error

rate if the input signal is weak and/or noisy.

Internally, the first stage of the decoding is

done by measuring the distance between the zero

crossings of the input signal in order to get a

raw binary stream.

If any of these distances are too short, we can

assume that an error occured.

noise-smoothing-threshold=value

-------------------------------

This defines the threshold value for the

noise smoothing feature.

The value defines the minimum length (in percentage)

that a zero transition distance must have to be considered

valid.

Example:

Sample rate is 48000 and the bitrates the program is configured

to decode is 512,1200 and 2400.

In this case, 2400 is the highest bitrate.

At this bitrate, one bit at 48000 hz is 20 samples long.

The zero transition will be considered valid if

it's greater than or equal to 20*0.value

If value=60, then 20*0.6 = 12

Another way of putting it:

The zero transition will be considered valid if

it's greated than or equal to

(sample_rate/highest_configured_bitrate)*0.value

If it's less than this, the noise smoothing code

will merge it with the adjacent transitions.

Default value: 60

charset=filename

----------------

Selects a character set.

Default value: charset.tbl

The included file "charset.tbl" is

intended for Swedish characters.

You can easily change it to suite your own needs.

1.5 Output modules

------------------

The following output modules are available:

console, external, file, mail, mysql, irc, telnet, winmsg

console:	Self explanatory.

external:	Executes an external application/script

to process decoded messages.

file:		Self explanatory.

mail:		Uses a sendmail compatible program

to forward messages.

mysql:		MySQL logging. Please read mysql-notes.doc.

irc:		Its original intention was to connect

to a TCL script running on an eggdrop IRC bot

to send decoded messages to IRC channels.

A TCL script for doing this is not included

with this program.

telnet:		Sends decoded messages to clients connected

to the telnet server.

winmsg:		Forwards messages to the messenger service

on Windows machines by using samba as backend.

This feature works on Windows 2000 and XP.

Windows 7 and Vista - not yet (?).

To use these, you'll have to provide the option "enabled=yes"

either in the configuration file or on the command line.

The exception from this is "file" and "console", as these

are enabled by default.

1.6 Output configuration

------------------------

The options for these are specified under [name] in

the configuration file.

It is also possible to specify the options on the command

line in the format

--name,option=value

It is possible to specify parameters containing space characters

by prepending them with "\".

The following options are specific to each output module:

console:	enabled=(yes|no)

Default: yes

eh-enabled=(yes|no)

Default: yes

external:	enabled=(yes|no)

Default: no

external=ID=program1,/path/to/program2

OR

external=program1,/path/to/program2

program1 and program2 can be any kind

of executable that takes command line

parameters.

The decoded message will be passed in the

format:

"Date" "RIC" "RIC lookup" "Channel" "Function bits" "Message"

Please see the included programs

"external.pl","external.sh" and "external.c"

for usage hints.

file:		logfile=ID=filename1,filename2

OR

logfile=filename1,filename2

Where the filenames can be either plain filenames,

or they can be in strftime format.

This module already has one logfile defined by default,

so it is not necessary to explicitly configure one.

enabled=(yes|no)

Default: yes

mail:		to=mail1,mail2

OR

to=ID=mail1,mail2

from=senderemail

Default: none (must be specified)

mta=path to sendmail or sendmail

compatible program.

Default: /usr/sbin/sendmail

enabled=(yes|no)

Default: no

mysql:		db=dbstr1,dbstr2

OR

db=ID=dbstr1,dbstr2

Where dbstr is in the format:

database_name:table_name:username:password:hostname:port

enabled=(yes|no)

Default: no

irc:		host=string1,string2

OR

host=ID=string1,string2

Where the string is in the format:

hostname:ip:#channel

enabled=(yes|no)

Default: no

eh-enable=(yes|no)

Default: yes

telnet:		port=ID=string1,string2

OR

port=string1,string2

Where the string is in the format:

port:password

Where "port" is a port the telnet server

should listen to, and "password"

the password needed to login on that port.

A password of "*" means that no password

will be required to connect.

winmsg:		host=ID=string1,string2

OR

host=string1,string2

Where the string is either an IP address

or the NetBIOS name of a Windows machine

which is resolvable by samba.

nmblookup=path to sambas nmblookup

Default value: /usr/bin/nmblookup

smblookup=path to sambas smbclient

Default value: /usr/bin/smbclient

The following option is common to all output modules:

discard-errored-messages=(yes|no)

Drops messages that contain any kind of decoding error.

An error defined as an ECC error (correctable or not)

in either the address codeword or the message codewords,

or both.

Default value: no

1.7 Filters

-----------

It is possible to define very specific filters for each output module.

Each filter has a policy and one or more rules and outputs associated

with it.

The following options are used with the filter function:

message-policy=(allow|drop)

message-policy=ID=(allow|drop)

ric-whitelist=string

ric-whitelist=ID=string

ric-blacklist=string

ric-blacklist=ID=string

keyword-whitelist=string

keyword-whitelist=ID=string

keyword-blacklist=string

keyword-blacklist=ID=string

function-bits-whitelist=string

function-bits-whitelist=ID=string

function-bits-blacklist=string

function-bits=blacklist=ID=string

keyword-case-sensitive=(yes|no)

keyword-case-sensitive=ID=(yes|no)

Where "string" is in the format

str1,str2,str3

Where each substring is in the format "ch:str" OR "str"

Example:

Let's say you only want to log messages addressed to the RIC 12345 to disk.

You'd put this in the configuration file (or the command line equivalent):

[file]

message-policy=drop

ric-whitelist=12345

We don't have to define an output for file because it already has one by

default.

Now let's say you want a slightly more complicated filter setup:

1. Log only messages to RIC 12345 as in the previous example.

2. Log everything except RICs 4444 and 5555.

To accomodate rule 2, we must assign that filter to its own ID, as follows:

[file]

message-policy=drop

ric-whitelist=12345

message-policy=1=allow

logfile=1=our_logfile.log,our_second_logfile.log

ric-blacklist=1=4444,5555

Now, only messages to 12345 will be logged to the default logfile,

and all messages except for those to 4444 and 5555 will be logged

to the files "our_logfile.log" and "our_second_logfile.log".

It is also possible to filter based on which channel a message comes in on.

Let's add a third rule to our configuration:

1. Log only messages to RIC 12345 as in the previous example.

2. Log everything except RICs 4444 and 5555.

3. Log only messages containing the keywords "error" or "warning"

coming in on channel 1.

[file]

message-policy=drop

ric-whitelist=12345

message-policy=1=allow

logfile=1=our_logfile.log,our_second_logfile.log

ric-blacklist=1=4444,5555

message-policy=2=drop

logfile=2=errors_and_warnings.log

keyword-whitelist=2=1:warning,1:error

The logic is:

For every output, lookup the policy assigned to that ID.

If drop:	Check if the message matches any of the whitelisted

characteristics.

If it does, process the message.

If allow:	Check if the message matches any of the blacklisted

charecteristics.

If it does, drop it.

The filters defined for each output module are completely independent of each

other.

1.8 Error highlighting

----------------------

This is a feature that allows you to see in greater detail

how decoding errors affect the messages.

For each alphanumerically decoded character from a POCSAG message,

the following conditions exist:

(1) It originated from one or two error-free codeword(s).

(2) It originated from one error-free codeword and one

codeword with one or two correctable error(s).

(3) It originated from one or two codewords with correctable

errors.

(4) It originated from one error-free codeword and one

codeword with uncorrectable errors(s).

(5) It originated from one codeword with correctable

errors and one codeword with uncorrectable errors.

(6) It originated from one or two codeword(s) with

uncorrectable errors.

For each of these conditions, the following colors are used in the error

highlighting:

(1) None

(2) Light green

(3) Dark green

(4) Yellow

(5) Yellow

(6) Red

For numerical messages, where characters don't cross codeword boundaries,

the equivalent conditions are:

(1) It originated from one error-free codeword(s).

(2) It originated from one codeword with one

or more correctable errors.

(3) It originated from one codeword with

uncorrectable errors.

Colors used in this case are:

(1) None

(2) Dark green

(3) Red

Currently this feature is implemented in the "console" and "irc"

output modules.

You can enable them with

eh-enable=yes

in each respective section.

1.9 Usage hints

---------------

* Input signal should be of a nice square wave shape (such as

from a discriminator tap on a scanner). Using the headphones output

is not reliable.

* Use the "screen" utility if you want to leave it running

in the background.

2.0 Known issues

----------------

* Endianness:

The sound input code assumes that it's running on a little-endian

system. If you want to run it on a big-endian system, you'll either have to

change "AFMT_S16_LE" to "AFMT_S16_BE" OR change the sample format to 8 bits:

dsp-sample-bits=8

The statistics file is not endianness-aware. That means you can't move the

file between LE and BE systems without converting it.

These issues may be fixed in a future release.

2.1 Contact

-----------

Send comments, suggestions, feature requests

and bug reports to: upocsag@s-lab.se

2.2 Acknowledgements

--------------------

I would like to thank the following people.

Their work has, in various ways, inspired and helped me create this program.

* Deti Fliegl

For creating my favorite POCSAG decoder for Windows (POC32).

* Robert Larsson

For his (now gone) pages on pager modifications and data decoding

(POCSAG and various other protocols).

* Clive Cooper

His program WinPE (Windows POCSAG Encoder) inspired me to

study the POCSAG protocol which ended up in me creating my own

POCSAG encoder (never released).

* Stefan Petersen

Created a very simple POCSAG encoder in C.

I used a snippet of that code (BCH) from that program

in my own encoder and also in this decoder.

* Robert Morelos-Zaragoza

For his bch3121.c

Error correction code.

2.3 License

-----------

This program is released under the GNU General Public License (GPL).

Please see the file COPYING for more details.

2.4 Legal stuff

---------------

I encourage an ethical and responsible use of this program.

You should be aware of the laws in your country that may

concern the usage of this program (such as the interception

of radio messages not directed at the public).